3.1 Dipole antenna pattern

The gain pattern of the short dipole is 1.5cos2φ, where φ is the angle in the x-z or y-z plane. Plot the gain pattern in the x-y vs. z plane, as in Figure 3.1. Find the half-power bandwidth.

Sol: The plot is

cos2φ=0.5φ=±π4

Therefore, the half-power bandwidth is π2.

3.2 Noise figure

The noise figure, denoted by F is defined as

F=Measured noise power out of device at room temperaturePower out of device if device were noiseless.

Explain why the output noise power described in Example 3.1 is NFG. Suppose the receiver consists of a cascade of three devices. Derive the composite noise figure.

Sol: If the device were noiseless, there would be non-zero output, which comes from the noise at the input. The power of this noise output is NG. Thus, the output noise power at room temperature is NFG. Each noisy device can be regarded as a combination of a noiseless device and a additive noise component:

The noise power of the additive noise is thus NFG-NG=NG(F-1). The cascade of the three devices can be regarded as

At the output of the first device, the noise power is NF1G1. The noise power at the input of the last device is

NF1G1G2+NG2(F2−1).

The noise power at the final output is

G3(NF1G1G2+NG2(F2−1))+NG3(F3−1)NF1G1G2G3+NF2G2G3−NG2G3+NF3G3−NG3

The noise power at the final output if the devices were noiseless is NG1G2G3. Thus, the noise figure is

NF1G1G2G3+NF2G2G3−NG2G3+NF3G3−NG3NG1G2G3=F1+F2−1G1+F3−1G1G2

3.3 Shadowing

Suppose the average propagation loss at the distance of 1km from the transmitter is 80dB. The average propagation loss is inversely proportional to the square of the distance. The shadowing effect is modeled as log-normal shadowing with σ=8dB. Compute the probability that the propagation loss at a distance of 10km is less than 90dB.

Sol: The formula for the propagation loss is

PL(d)[dB]=PL(d0)+10nlogdd0+Xσ,

where Xσ is a random variable with a normal distribution. After plugging the numbers into this formula, we obtain

PL(10km)[dB]=80+20+Xσ.

The probability is Q((100-90)/8)=0.1056.

3.4 Coverage area

The coverage area U(γ) is defined as the percentage of useful service area (i.e. the percentage of area where the received signal is equal to or greater than γ), given a known likelihood of coverage at the cell boundary (boundary coverage, i.e. the fraction of the time the received signal power Pr is to exceed the threshold γ at the boundary R). The formula for the boundary coverage is

P[Pr(R)>g]=12−12erfg−[Pt−(PL(d0)+10nlog(Rd0))]s2

If we let a=(γ−Pt+PL(d0)+10log(Rd0))σ2 and b=(10nloge)σ2, the coverage area is

U(γ)=121−erf(a)+exp1−2abb21−erf1−abb.

Plot the curves of U(γ) from σn=0 to 8 for boundary coverage of 0.5, 0.55, …, 0.95. Also comment on the plot.

Sol: U(γ) is a function of a and b. b is directly a function of σn whereas U(γ) and the boundary coverage are related by a. The boundary coverage is also a function of a. Thus, we have to solve a for every value of boundary coverage. The plot is given below

It is observed that with higher boundary coverage the coverage area increases. Besides, the channel fading is harmful to the communications. With higher σn, the channel is more likely in deep fading and thus the area coverage and boundary coverage drop.

3.5 Satellite link budget

Suppose a satellite receiver consists of a low noise amplifier with G=200 and F=1.5, with subsequent stages having the noise figure 30. The satellite is in geostationary orbit(r=40,000km), the carrier frequency is 7.5GHz, the bandwidth W is 5kHz, Gt=200, Gr=100, and the required SNR is 10 (all in absolute units) for 90% of time when the rate of rain fall is 50mm/hr and the temperature is 300K. The rainstorm path length is 5km. The standard deviation  in the log-normal distribution is 10dB. Compute the required transmission power.

Sol: N0=2.07×10−17, or –166.8dB. F=1.5+29/200=1.645 or 2.16dB. The carrier frequency of 7.5GHz corresponds to a wavelength of 3×1087.5×109=0.04. Thus PrPt=200×100×(0.04(4π×4×107))2=1.27×10−16 or –158.97dB. That is, Pr=Pt−158.97dB. The loss due to rain fall is 0.00481×50×5=1.2025 or 0.8dB. The margin for 90% coverage is 8.42dB. Thus, the result is

Pt(dB)=158.97+F+N0+SNRreq+Margin+Rainfall loss158.97+2.16-166.8+10+0.8+8.4213.55(dBW) or 22.65W

3.6 Frequency coherence

For a WSS process, prove that

fC(f1,f2;Dt)=-¥¥fct;Dte−j2pDftdtºfC(Df;Dt)

Sol: Since it is a WSS process

φC(f1,f2;Δt)≡12E[c∗(τ1;t)c(τ2;t+Δt)]ej2π(f1τ1−f2τ2)dτ1dτ2=φc(τ1;Δt)δ(τ1−τ2)ej2π(f1τ1−f2τ2)dτ1dτ2=φc(τ1;Δt)ej2π(f1−f2)τ1dτ1=−∞∞φcτ;Δte−j2πΔfτdτ≡φC(Δf;Δt)

By substituting the definition of the autocorrelation function, we obtain

φC(f1,f2;Δt)=φc(τ1;Δt)δ(τ1−τ2)ej2π(f1τ1−f2τ2)dτ1dτ2

If we integrate the delta function, we obtain

φC(f1,f2;Δt)=φc(τ1;Δt)ej2π(f1−f2)τ1dτ1=−∞∞φcτ;Δte−j2πΔfτdτ≡φC(Δf;Δt)

3.7 Doppler spectrum

Consider a mobile unit with an antenna characterized by gain G(θ). If the distribution of the power of the reflected signal is p(θ), then within a differential angle dθ, the normalized received power is G(θ)p(θ)dθ. This power is equal to the differential variation of the received power with frequency, S(f)df, where S(f) is the Doppler spectrum. Determine and plot S(f ) for the transmitted unmodulated carrier when an omnidirectional antenna is used with G(θ)=1, θ is uniformly distributed, and the total power of the received signals is 2σs2.

Sol: Noting the Doppler frequencies from signals from θ and −θ are the same, we have

S(f)|df|=G(θ)p(θ)+G(−θ)p(−θ)|dθ|.

The Doppler frequency fi(θ) is related to the carrier frequency fc, the mobile unit speed v, the speed of light c, and the incident angle θi through

fi(θ)=fc−vfcccosθi.

Denoting fd=vfcc, we get

|df|=fd|sinθdθ|=fd2−(f−fc)2|dθ|.

Thus, the spectrum is

S(f)=p(θ)+p(−θ)fd1−f−fcfd2fc−fd≤|f|≤fc+fd0otherwise

To compute p(θ), we note that p(θ) is a constant and the integration of S(f ) must be equal to 2σs2. Thus, The power distribution is

p(θ)=2σs22πfd.

The Doppler spectrum is

S(f)=2σs22πfd1−f−fcfd2−12fc−fd≤|f|≤fc+fd0otherwise

The plot of the Doppler spectrum is

3.8 Sonar

Consider two submarines under water. Submarine A transmits an acoustic signal to determine the speed of submarine B in the direction towards A. The frequency of the transmit signal is 100Hz. The frequency of the received signal after being reflected from submarine B is 101Hz. The speed of submarine A in the direction to submarine B is 5m/s w.r.t. the land. The temperature of the water is 288K. There is also a very fast water current as depicted in Figure 3.14. The speed of water current is 1m/s. Compute the speed of submarine B w.r.t. the land.

Figure 3.14  Submarine tracking

Sol: The speed of sound in water at 288K is 1450m/s. Since the water is moving, the speed of sound from A to B is 1450+1×cosπ3=1450.5ms. Denoting the speeds of submarine A and B as vA and vB, then the acoustic frequency received by submarine B is

fB=100×1+vB1450.51−vA1450.5.

The acoustic signal is reflected by submarine B with the same frequency fr. The speed of sound from B to A is 1449.5m/s. The reflected signal is received by submarine A at frequency

fA=fB×(1+vA1449.5)=fB×(1+5/1449.5)=101.

Thus, we get vB=4.4362ms.

3.9 Geological layers

Consider Example 3.12. The distances to the detectors are x1=100m and x2=200m. The propagation times to node 1 and node 2 are 0.6675 s and 0.67 s respectively. Compute the values of d and v.

Sol:

0.6675=2r1v=2(x12)2+d2v0.67=2r2v=2(x22)2+d2v

Therefore,

(x12)2+d20.66752=(x22)2+d20.672⇒d=998.44m.

v can also be obtained

v=2r10.6675=2995.3ms.

3.10 Image location

Consider two glass lenses A and B with unknown radii of curvature. Nevertheless, the two lenses are known to have the same curvature on one side. When lens A is tested and rays propagate from the side of the identical curvature at the distance of 20cm , the image is at the distance of 40cm. When lens B is tested and rays propagate from the side of the identical curvature at the distance of 80 cm, the image is at a distance of -40cm. Determine the location of the image when two lenses are merged together along the identical curvature side and rays propagate from the side of lens A at a distance of 30cm.

Sol:

Note that two thin lenses, when put together, form a new thin lens.  Here also, given the differences in image location the way they fit together along the sides of identical curvature is that one curvature is convex while the other is concave.  From the first experiment, we know

0.5×1r1+1r2=10.2+10.4,

where r2 is the radius of the identical curvature and r1 is the radius of the other curvature. From the second experiment, we know

0.5×−1r2+1r3=10.8−10.4,

where r2 is the radius of the identical curvature and r3 is the radius of the other curvature. We want to find the location of image, denoted as i, in

10.3+1i=0.5×1r1+1r3.

The right side is exactly equal to the summation of the first two equations. Thus,

10.3+1i=10.2+10.8,

and i=12/35m.

3.11 Near-ground radio propagation

In free space, the electric field received by a receiver antenna which is separated from a radiating transmitter antenna by a distance d is

E(d,t)=E0d0dcosωct−dc.

The received power is given by the Friis free space equation

Pr(d)=PtGtGrλ2(4π)2d2.

In the two ray ground reflection model as depicted in Figure 3.15, the received electric field is

ETOT(d,t)=E0d0d'cosωct−d'c−E0d0d} } cos left (ω rSub { size 8{c} }  left (t -  {  {dc

Figure 3.15  Near-ground radio propagation

The T-R separation distance d is very large compared to hr and ht. Derive the received signal power.

Sol: The path difference , Δ, can be expressed as

Δ=d - d'= sqrt { \( h rSub { size 8{t} } +h rSub { size 8{r} }  \)  rSup { size 8{2} } +d rSup { size 8{2} } }  -  sqrt { \( h rSub { size 8{t} }  - h rSub { size 8{r} }  \)  rSup { size 8{2} } +d rSup { size 8{2} } }  approx  {  {2h rSub { size 8{t} } h rSub { size 8{r} } }  over  {d} } } {.

It is also noted that

E0d0d≈E0d0d'≈.

The phase difference θΔ is

θΔ=2πΔλ=Δωcc.

At t=d
            c} {, the electric field can be expressed as

ETOT(d,t=d
                      c \) = {  {E rSub { size 8{0} } d rSub { size 8{0} } }  over  {d'} } cos left (ω rSub { size 8{c} }  left ( {  {d−d'c−E0d0d} } cos0 rSup { size 8{ circ } } }  {} # 
= {  {E rSub { size 8{0} } d rSub { size 8{0} } }  over  {d'} } cosθ rSub { size 8{Δ} }  -  {  {E rSub { size 8{0} } d rSub { size 8{0} } }  over  {d≈E0d0d[cosθΔ−1]

Referring to the phasor diagram which shows how the direct and ground reflected rays combine,

the electric field at the receiver can be written as

|ETOT(d)|=E0d0d2[cosθΔ−1]2+E0d0d2sin2θΔ2E0d0dsinθΔ2≈2E0d0d2πhthrλd

Compared with E(d,t) and Pr(d) for the free space propagation without reflection, the received power at a distance d from the transmitter can be expressed as

Pr=PtGtGrht2hr2d4.