12.1 Sensor density and resources

(a) Qualitatively, how does increasing sensor density reduce the error in reproducing a source at a fusion center?

(b) Instead of increasing sensor density, an administrator decides to provide deterministic mobility to the sensors. Having deterministic mobility means that there is no randomness in the motion of sensors and the administrator can control the motion. How will the provision of mobility help in reducing the error at the fusion center?

Sol:

(a) By increasing the sensor density, the number of sensors observing the source increases. With more sensors measuring the same source, the discrepancy of reproducing the source at the fusion center decreases. Also, the sensors get closer to the source and hence enhance the SNR available at the sensors. Due to these reasons the error in reproducing the source decreases.

(b) If the sensors are able to move as needed then it serves the same purpose as increasing density. For example, the sensors observing the source can be moved closer to the source and hence improve the quality of measurement. In some cases, this is even better than increasing sensor density since a sequence of measurements from different perspectives can be obtained at very fine spatial resolution.  However, as measurements are not taken simultaneously in all these positions there are still good reasons to also have multiple fixed sensors.  Both approaches provide diversity, with it being easier to provide dense spatial sampling with mobility, and dense temporal sampling with fixed sensors.

12.2 Network throughput with mobile nodes

Consider a network with n mobile sensors. The source nodes transmit the message to the corresponding destination pair in two steps. In step 1 the source transmits the message to the relays or destination; and in the next step relays forward the message to the destination. If there is no constraint on delay then what will be the order of network throughput for this scheme?

Sol:

Since there is no constraint on delay, the relay can deliver the message to the destination only when it gets closer to the destination. As there are n nodes, one can get network throughput of O(n) from this scheme.

12.3 Work, force and power

It is required to lift a sensor node from ground to the height of 40 m in 12 s. This work requires 950 J.

(a) What amount of force and power is required to lift the sensor?

(b) Calculate the mass of the sensor.

Sol:

- From the work equation, F=W/d =  23.75N. The power is given by W/t =  79.17 W.

- Now, W=mgh. Therefore the mass of the sensor node is, m = W/gh = 2.42 kg.

12.4 Nodes in freefall

A sensor node is to be dropped from the height of 15 m, sending out readings as it falls. How much total energy is possessed by a 2 kg sensor node if it is dropped at the velocity of 2 m/s? Also, calculate the final velocity of the falling sensor node just before it reaches ground.

Sol:

ET = Total Energy = Potential Energy (due to position) + Kinetic energy (due to motion)

= mgh + (1/2)mv2 = (2  9.8  15) + (0.5  2  22) = 298 J.

The final velocity can be obtained from:

(1/2)mv2 = ET = 298 J

Therefore, the final velocity neglecting atmospheric drag is v = 17.26 m/s.  The node had better be tough!

12.5 Stepping up

A 7.5° stepper motor accelerates from zero to 45.84steps/s in 1 ms. If the rotor and load inertia, J, is 210-3 g m2, calculate the required torque.

Sol:

Number of steps/revolution, S = 360°/7.5° = 48 steps/rev.

The acceleration in terms of rad/s2, = (∆v/∆t)  (2/S) = 6000.44 rad/s2

Therefore, torque, T = J = 12 N m2

12.6 Effects of environmental obstacles

Consider a sensor field deployed at uniform density, d, which yields an intersensor spacing, a, in a regular grid deployment. Let the area for which one sensor is responsible be denoted by A=a2. Consider a circular obstacle of diameter, l, present which blocks the coverage of the sensor. Suppose the area occluded by the obstacle is A0. Assume that the sensor is capable of limited mobility over a range that is a small multiple of l. Define the probability of missed detection as

Pmiss=1AA0ftarget(x,y)dxdy

where ftarget(x,y) is the probability density of the target location within A.

Assume ftarget(x,y) to be uniform. Consider the position of the obstacle shown in Figure 12.17 such that both the tangents from the sensor to the obstacle circle meet the top edge of the tile. Let 1 be the angle made by the tangent to the obstacle edge with the vertical line joining the sensor and the center of the obstacle. Let the distance of the obstacle center from the sensor be x.

(a) Calculate Pmiss for this scenario.

(b) If the sensor moves from the present location to 2l, what is Pmiss?

Figure 12.17 Occlusion geometry

Sol:

(a) Here,

A = a2 - (l/2)2,

and, A0 = a2 tan1 – (l2/4tan1) – ( + 1) (l2/8) where 1 = sin-1(l/2x)

Since ftarget(x,y) is uniform, Pmiss = A0/A.

(b) Here A will remain as in (a), but A0 will change and can be calculated to be:

A0 = (l2/4) tan -  (l2/4)

where,

 = (/2 + 1 + 2 + )/2

with,

q2=sin−1124l2+x2β=tan−1(x2l)

12.7 Gain from mobility

The gain from mobility, G, can be characterized as the ratio of Pmiss(static sensor node) to that of Pmiss(mobile sensor node). Using the same set-up as Problem 12.6, let a=100, l  (5, 20) and the distance of the obstacle, x  (20, 35). Plot the curves of gain versus obstacle distance for l in steps of 5.  What conclusions may be drawn?

Sol:

The figure clearly shows that small mobility reduces the occluded area by about an order of magnitude in the above cases.

12.8 Shadow Area

In Figure 12.18, cameras with a 180° field of view are denoted by triangles and occlusions by dark squares.  Mark and calculate the area of the shadow region.

Figure 12.18  Obstacles and cameras

Sol:

The lines defining the shadow region are drawn.  Four squares are completely shadowed, while several others are partially occluded.  The total area obscured is a little less than 7 squares.

12.9 Exploiting gaps

Repeat example 12.16 for Figure 12.19

Figure 12.19 Camera path and obstacles

Sol:

The squares that are largely shadowed are indicated

12.10 The data mule track

Consider the four nodes sensor network as shown in Figure 12.20. Node G is the data sink.  Communications is by means a data mule.  Movement of one unit costs 3 W h.

(a) Find the optimal path for the data mule assuming the mule must be in close proximity to each sensor to transfer data.

(b) Now suppose that information mule must approach within 2 units distance of the nodes A and C and 1 unit distance of the node B to attain the desired communications. Assuming negligible communications energy cost, find a circuit with near-minimum cost, and compute that cost.

Figure 12.20 Information source/sink layout

Sol:

- In this case the optimal circuit is clearly GACBG or the reverse direction, with a cost of 3(5+26+3+20)=52.7 W h.

- Suppose G to be at (0, 0). Clearly we can do better than following the path in (a) with the given constraints. Construct circles of 2 units as A and C as center and 1 unit with B as center. The mule travels from G to point P (2, -3) to exchange information with B and from P to Q (3.2, -3) to exchange information with C. From Q, the close to minimum travel distance (and hence energy) would be to R (2.6, -0.5) to exchange information with A. Hence, with the constraints given, the data mule should travel along the quadrilateral GPQR yielding 30.09 W h.