8.1 Connectivity

Connectivity is usually used to describe the reliability of the network. In a network, if there is a path connecting nodes i and j after removing (k-1) other nodes and their associated links, then i and j are said to be k-connected. A network is said to be k-connected if any pair of nodes are k-connected. Therefore, a k-connected network can survive (k-1) node failures. Another notion of interest is arc connectivity. It is defined as the minimum number of links needs to be removed in order to disconnect the network. In other words, a k arc-connected network can withstand at least (k-1) link failures.

  - Determine the node and arc connectivity of star, tree, ring, and cellular network topologies.

  - Determine the node and arc connectivity of the following two ad hoc topologies, shown in Figure 8.26.

Figure 8.26 Two ad hoc network topologies.

Sol:

The node and arc connectivities of various networks are given in the following table.

| Network topology | Node conectivity | Arc conectivity |
| --- | --- | --- |
| Star | 1 | 1 |
| Tree | 1 | 1 |
| Ring | 2 | 2 |
| Cellular | 1 | 1 |
| (a) | 1 | 2 |
| (b) | 2 | 2 |

8.2 Diameter

The distance between a pair of nodes in the network is the length of the shortest path connecting the two nodes. In the case of disconnected nodes, the distance is set to infinity. The diameter of a network is then defined as the maximum distance of any pair of nodes in the network. Determine the diameter of following network topologies assuming all the links have weight one:

- a star network;

- a ring network consisting of n nodes;

- a cellular network with three fully connected base stations;

- the tree network in Figure 8.27;

- the two ad hoc networks in Figure 8.26.

Figure 8.27 A tree network topology.

Sol:

- The diameter of a star network is 2.

- For a ring network with n nodes, the diameter is n/2 when n is even and (n-1)/2  when n is odd.

- The diameter is 3 for this cellular network.

- The diameter is 5 for the nework in the figure.

- The diameter is 3 for both topologies.

8.3 Flooding

Flooding is often used in a local network to update path state information. To limit the number of transmissions, two rules are observed during flooding: the node will not relay the packet back to the sender; the node only relays the packet to its neighbor once. Consider the network topology in Figure 8.28. Assuming the transmission along each link takes time T, following the format given in Table 8.4, describe the network activity for the flooding initiated at node A. Assuming that point to point transmission is used, in other words no broadcasting, find the total number of transmissions that have occurred during the flooding.

Figure 8.28 Flooding in network.

| Time | Transmission (sender  reciepent) |
| --- | --- |
| [0, T] | A B |
| [T, 2T] | B C, BD |
| … | … |

Table 8.4 Network activity during flooding.

Sol:

The network activity is described by the following table.

| Time | Transmission (sender  reciepent) |
| --- | --- |
| [0, T] | A B |
| [T, 2T] | B C, BD |
| [2T, 3T] | CG, DE, DF |
| [3T, 4T] | GE, GH, EG, EF, FE, FH |

A total of 12 point to point transmissions have occurred.

8.4 Shortest path routing: Bellman-Ford algorithm

In this problem, we will use the network topology in Figure 8.29 to illustrate the Bellman-Ford algorithm for finding the shortest route to a node. In the figure, the number beside a node serves as the label for the node, and the number near an arc indicates the length of the arc. For instance, the arc connecting nodes 1 and 2 has length 1. Define dij to be the length of the direct arc connecting nodes i and j. If there is no direct arc connecting the two nodes, we set dij=∞. By doing this, dij has meaning for any pair of nodes i and j in the network.

Figure 8.29 A simple network.

Consider node 1 as the destination node. The shortest path from node i to node 1 that traverses at most h arcs and goes through node 1 only once is called a shortest (h) walk, and its length is denoted by Dih. Note there are two special cases. If all paths between node i and 1 consist of more then h arcs, Dih=∞. By convention, D1h=0 for any h.

- Determine Di0 for i=1, 2, …, 6. Find dij for all possible i, j=1, 2, …, 6.

- The following iteration is used to generate the subsequent shortest walks.

Dih+1=minj[dij+Djh],       for all i≠1.

Determine Di1 for i≠1.

- Use the iteration equation in (b) to compute Di2, Di3, … for i≠1. Stop the iteration when

Dih+1=Dih,     for all i≠1.

The minimum distance from node i to node 1 is Dih in the last iteration.

Sol:

- It is easy to see

Di0=0i=0∞otherwise

And dij is described by the following matrix, in which the element at ith row and jth column is dij.

dij=015∞∞∞101∞10∞5104∞∞∞∞4013∞10∞301∞∞∞310

- From what we have in (a), we can determine

Di1=1i=25i=3∞i=4,5, and 6.

- Continue the iterations, for h=2 going forward:

| h | D1h | D2h | D3h | D4h | D5h | D6h |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 5 | ∞ | ∞ | ∞ |
| 2 | 0 | 1 | 2 | 9 | 11 | ∞ |
| 3 | 0 | 1 | 2 | 6 | 10 | 12 |
| 4 | 0 | 1 | 2 | 6 | 7 | 9 |
| 5 | 0 | 1 | 2 | 6 | 7 | 8 |
| 6 | 0 | 1 | 2 | 6 | 7 | 8 |

The minimum distance from node i to node 1 is determined when h=6, since there is no change in the table from the previous iteration.

8.5  Shortest path routing: Dijkstra algorithm

We will use Figure 8.29 to illustrate the Dijkstra algorithm for finding the shortest route to a destination node. The length of the direct arc connecting nodes i and j is defined to be dij. For a detailed description of the figure and the definition of dij, please refer to Problem 8.4. Denote by P the set of nodes whose shortest path to the destination node is known, and denote by Dj the current shortest distance from node j to the destination node. Note that only when node j belongs to the set P can we say Dj is the true shortest distance. Choose node 1 as the destination node. Initially, set P=1, D1=0, and Dj=∞ for j≠1.

- Update Dj for j≠1 using the following equation

Dj:=min[Dj,dj1]

- Find i such that

Di=minj∉P[Dj]

Update P:=P∪i.

- Update Dj for j∉P by the following equation

Dj:=min[Dj,Di+dji]

in which i is the i obtained in (b).

(d) Go back and compute steps (b) and (c) recursively until P contains all the nodes in the network. The resulting Dj is the shortest distance from node j to node 1.

Sol:

The iteration is illustrated in the following steps.

Step 1:

Dj=1j=25j=3∞j=4,5, and 6.

i=2, P={1, 2}

Step 2:

Dj=2j=311j=5∞j=4 and 6.

i=3, P={1, 2, 3}

Step 3:

Dj=6j=411j=5∞j= 6.

i=4, P={1, 2, 3, 4}

Step 4:

Dj=7j=59j=6

i=5, P={1, 2, 3, 4, 5}

Step 5:

Dj=8j=6

i=6, P={1, 2, 3, 4, 5, 6}

8.6 DSR

For the network topology in Figure 8.30, assuming the same transmission delay on all the links, use the dynamic source routing algorithm to determine the routes from node 1 to node 12,  from node 3 to node 12, and from node 6 to node 8.

Figure 8.30 DSR.

Sol:

The possible routes are listed.

From node 1 to node 12:	1261012.

From node 3 to node 12:	31261012

35891112

35471112

35471012

Form node 6 to node 8:	6798

6748

6248

8.7 Spanning tree

Find all possible spanning trees for the two graphs in Figure 8.31 subject to the constraint that node 1 must be the root. Determine the number of nodes N and arcs A in each of these spanning trees. Can you see a relation between N and A?

Figure 8.31 Spanning tree.

Sol:

- There are 3 spanning trees with node 1 as the root.

N=4, A=3.

- There are 5 spanning trees with node 1 as the root.

N=5, A=4. It can be seen that the following relation holds: N=A+1.

8.8 Directed diffusion

Consider the situation in Figure 8.32, which is similar to Example 8.6. The solid lines represent transmission links between nodes, and dashed lines indicate boundaries of tiers.  Here node A wants to transmit to node D. Suppose the transmission takes the branches within the same tier with one third of the probability of branches in the next tier, and the packets do not back-track. Determine the likelihood of packets flowing through node B and C to reach D.

Figure 8.32 Directed diffusion

Sol:

First compute the probability of choosing particular paths for data exiting a node.

The probability of transmission taking place on each branch is listed in the following figure.

Therefore the likelihood of transmission flowing through B and C are 241/1344 and 1103/1344  respectively.  Notice that there is bi-directional flow of packets along one link, and here one must invoke the rule against back-tracking to compute the output flows for the two nodes it connects.

8.9 M/M/1 queues

Consider the infinite length M/M/1 queue discussed in Example 8.7.

- Given the probability of n customers in the queue is P(n)=(1−ρ)ρn, where ρ=λμ, show that the average number of customers in the queue is

N=E(n)=n=0∞nP(n)=ρ1−ρ.

- Plot N as a function of ρ when 0≤ρ<1. What happens when ρ≥1?

- Find the average delay that customers experience and the average waiting time that customers spend in queue. (Hint: use Little’s theorem as in problem 7.12.)

Sol:

- The average number of customers in the queue is

N=n=0∞nP(n)    =n=0∞n(1−ρ)ρn    =(1−ρ)ρddρn=1∞ρn    =(1−ρ)ρddρρ1−ρ    =ρ1−ρ

- The plot is shown in the following figure. The average number of customers in the queue will become unbounded when ρ≥1.

- From Little’s theorem, the average delay experienced by customers is

D=Nλ=1μ−λ

The average service time experience by customers is 1μ. Therefore, the average waiting time is

W=D−1μ=λμ−λ

8.10 ARQ

Consider a simple ARQ scheme through a single transmission link of data rate R. The ARQ scheme works as follows. The sender transmits a data packet across the link. Once the receiver receives the whole packet, it checks if data have been corrupted. If there is no error, a packet is sent to the sender to acknowledge the correct receipt of the data packet. If there is an error, an ARQ is sent for a retransmission. The sender resends the packet immediately after it receives the ARQ packet. Assume the lengths of data and ARQ packets are L and LARQ respectively, and the propagation delay along the link is given as td. Neglect the turn-around time at the sender and the receiver. Suppose that the probability the data packet is corrupted during transmission is Pe and ARQ packets are always correctly received.

- Determine the average number of transmissions required for a packet to be correctly received.

- Find the average delay a packet experiences. The delay is defined as the time interval between the start of the first packet transmission and the end of the correct packet reception. Note it does not include the transmission of the last acknowledgement packet.

Sol:

- That the packet is correctly received after n transmissions is to say that there are n−1 corrupted transmissions and 1 correct transmission. The probability is thus given by

pn=(1−Pe)Pen−1

The average number of transmissions required for a correct reception is

N=n=0∞npn=n=1∞n(1−Pe)Pen−1    =(1−Pe)ddPen=1∞Pen    =(1−Pe)ddPePe1−Pe    =11−Pe

- Since there are a total number of N data packet transmission and N−1 acknowledgement packet transmissions, the average delay experienced by a packet is

D=LR+tdN+LARQR+td(N−1)

where N is the average number of transmissions from (a).

8.11 Source localization

Consider source localization using clusters of sensors. Shown in Figure 8.33 are the source and two clusters of sensors located at coordinates (0,0) and (1,0). Using the beamforming technique discussed in chapter 7, the two clusters of sensors are able to determine that the arrival angles of source waves are α and β as depicted in the figure.

(a) Determine the location of the source (x,y) as a function of α and β.

(b) Suppose there is small error Δα in the measurement of α. Determine the resulting error Δxinx. Comment on how the values of α and β affect Δx.

Figure 8.33 Source localization

Sol:

- From the geometry, we have the following equations:

yx=tanαyx−1=tanβ

The coordinates of the source can be found to be as follows:

x=tanβtanβ−tanαy=tanαtanβtanβ−tanα

- The error Δx resulting from Δα is

Δx≈dxdαΔα=tanβ(tanβ−tanα)2cos2αΔα=κΔα.

As can be seen from above formula, when α and β are about the same, the coefficient κ is large.  Consequently, Δx is sensitive to the measurement error Δα.

8.12 Channel Capacity – Gaussian channel

The channel capacity of a continuous Gaussian channel is given as

C=Blog21+SNB

in which S is the signal power, N is the noise power density, and B is the channel bandwidth.  Find the C when B→∞.

Sol:

Evaluate the channel capacity when B→∞.

C=limB→∞Blog21+SNB=limB→∞Bln1+SNB/ln2=limB→∞B⋅SNB⋅1ln2=SNln2

where we used log2x=lnx/ln2 and ln(1+x)≈x for x<<1.

8.13 Slepian-Wolf coding

Consider two discrete random variables X and Y. Their joint distribution is given in Table 8.5.

- Compute H(X), H(Y), H(X|Y), H(Y|X), and H(X,Y).

- Supposing X and Y are observed and coded by two sensors, compare the source rates with independent and Slepian-Wolf coding strategies.

Table 8.5 Joint distribution of X and Y

X	Y	P(X,Y)

0	0	1/2

0	1	1/6

1	0	0

1	1	1/3

Sol:

- From the joint distribution, we can deduce the following.

P(X=0)=23,      P(X=1)=13

P(Y=0)=12,      P(Y=1)=12

The entropies can be computed

H(X)=−xP(x)log2P(x)=0.9183

H(Y)=−yP(y)log2P(y)=1.0

H(X,Y)=−x,yP(x,y)log2P(x,y)=1.4591

H(X|Y)=H(X,Y)−H(Y)=0.4591

H(Y|X)=H(X,Y)−H(X)=0.5409

- If the independent coding strategy is used, a total rate of 1.9183 bits is required for coding X and Y. If Slepian-Wolf coding is used, the constraints on the rates are RX≥0.4591 bits, RY≥ 0.5409 bits, and RX+RY≥ 1.4591. Therefore, a total rate of 1.4591 bits is required.

8.14 Network capacity

Use R1 and R2 as two coordinates. Plot the rate region that is achievable in a two-user Gaussian multiple access channel assuming PN=1.

Sol:

As PN=1, the constraints on the network capacities are given by

R1<CPN=0.5R2<CPN=0.5R1+R2<C2PN=0.7925

These are plotted in the figure below.

The capacity region is indicated by the shaded region.

8.15 Rate-distortion

Consider rate-distortion coding with a single helper. Suppose σX2=1, DX=0.1. Plot the lower bound ofRX as a function of R1 for ρ=0, 0.4, 0.7, and 1. Comment on how this bound changes as R1 and ρ change.

Sol:

With a single helper, the bound of RX is given as

RX≥12log2σXDX1−ρ2+ρ22−2R1

This is plotted in the following figure for σX2=1 and DX=0.1. As can be observed from the plot, the bound on RX decreases as the covariance ρ and the auxiliary rate R1 increase.