10.1 Energy supply

In Example 10.1, a node uses a zinc-air battery (at 1V) instead of a lithium-ion  battery. How large must the battery be for reliable operation of the node for 15 days? Further, the weather is not friendly and provides only 8 h of daylight. How large must the photovoltaic cell be?

Sol:

There are 360 h in 15 days, and thus the zinc-air battery must supply 3600 mW-h. For the reliable operation, we should consider the power density of the battery to be 1050 mW-h/cm3 at 1.4V. Therefore, the power density at 1V is 750 mW-h/cm3. The volume must therefore be 4.8cm3. Since the sunlight is available only for 8 h, the battery must be charged to enable the operation for the remaining 16 h and hence 30 mW must be produced. Since the concern is reliability, assume cloudy conditions. Hence, the size of the array must be at least 30/0.15 = 200 cm2.

10.2 Cost of storing energy and service life

(a) An installation requires 60 cells in series to produce 120V. Depending on the application, this output is converted to different ac voltages. The cost of a 450 A-h, twelve cell battery is $3000. In order to the get the longest possible life, the batteries are float charged. Let us assume that the batteries are float charged at 2.25 V per cell. Calculate the cost of storing energy.

(b) To reduce the power consumption, a sensor is needed to sense the area once every 10 min for a month. What is the service life for the battery?

Note: service life = (total desired operational time of a sensor  cycles) / (sensing duration)

Sol:

(a) Cost of energy storing, C = (30005)/ (450120) = $277.78/kWh.

(b) Assuming 31 days in a month, the service life for a battery is:

L =  (31  24  60)/ 10 = 4464 cycles.

10.3 Dynamic power consumption in CMOS.

(a) Discuss the different ways of reducing power consumption in CMOS.

(b) What is the net effect on the power consumption in CMOS if the transition frequency is doubled and the voltage swing is halved?

(c) What is the effect of transistor size on power consumption for a fixed transition frequency?

Sol:

(a) Reducing C, Vdd, and f.

(b) Power consumption is halved.

(c) The device capacitance, C, is directly proportional to the transistor size. Hence with the increase or decrease in the sizes of transistors, the power consumption will respectively increase or decrease.

10.4 Parallelism, power and delay

How does parallelism affect power consumption and delay in circuits? Consider the power consumption and delay equations as in Example 10.3.

Sol:

There are two cases to consider: Vdd >>Vt or having it near Vt.  Without loss of generality, consider the case of two parallel processing streams rather than one.  In the former case, since there will be two parallel streams, for the same net delay we can cut the frequency in half, allowing reduction in also Vdd by roughly a factor of 2.  Thus the power in each branch is reduced a factor of 8, for a net savings of a factor of 4.  In general, with N parallel streams the savings will go as N2. When Vdd is near Vt, negligible net savings in power would result since even substantial increases in permitted delay represents a tiny decrease in Vdd. Thus the maximum benefit of parallelism is a power savings of approximately (Vdd -Vt)2.

10.5 Scheduling based on deadlines

Consider an application of a sensor network that demands a distributed processor platform.  We have 3 processors (P1, P2, P3), each having four modes of operation – boot-up mode (power consumption of 3W), execution mode (power consumption of 5W), idle mode, and shut-off mode (power consumption of 2W). Assume no power consumption for an idle mode. Also, the processors dissipate constant power for each time unit.  The boot up time and shut-off time for (P1, P2, P3) are (12 s, 8 s, 10 s) and (9 s, 6 s, 6 s) respectively. There are two applications that are needed to be executed utilizing all three processors. The first application needs to be run at t = 60s and the second application at t = {30 s, 60 s}. The execution time of each application is 10 s. The processors allow for parallel execution. For the minimum energy scheme:

(a) Give the start time of all the processors.

(b) Calculate the net energy on the platform for this scheme.

Sol:

(a)

| Processors | Application 2 (exe-1) | Application 1 & Application (exe-2) |
| --- | --- | --- |
| P1 | 18 sec. | Idle mode |
| P2 | 22 sec. | 52 sec. |
| P3 | 20 sec. | 50 sec. |

(b) Net Energy = Boot Energy (BE) + Execution Energy (EE) + Shut-off Energy (SE).

For the second application (first instance of execution):

BE1 = (312) + (38) + (310) = 90 J.

EE1 =  510		         = 50 J.

SE1 = (26) + (26)                  = 24 J (P1 remains in idle mode).

For the first application and second application (second instance of execution), P2 and P3 are booted while P1 remains idle. Therefore,

BE2 = (38) + (310)           = 54 J.

EE2 =  510                          = 50 J. (Parallel Execution)

SE2 = (29) + (26) + (26) = 42 J.

Therefore, the net energy consumption = BE1 + EE1 + SE1 + BE2 + EE2 +SE2

= 310 J.

10.6 Direct transmission vs. multi-hop.

Figure 10.5 N-hop linear network

Consider the scenario depicted in Figure 10.5. The N+1 nodes are placed in a linear topology. All adjacent nodes are spatially separated by distance r. The radios on the nodes consume E J/bit when turned on, in either transmit or receive mode. The radio front end consumes  J/(bit-mx) to amplify the signal, to arrive at a receiver 1m away, having a desired signal to noise ratio of s1 (where x is the order of power drop-off with distance). For both second order and fourth order power drop-offs:

(a) Derive the energy equations for the direct transmission and multi-hop scenarios.

(b) What is the condition for which direct transmission consumes less energy per bit than multi-hop transmissions.

(c) Plot the feasible region for direct communications in the r - N plane and comment on the consequences. For your plot, assume the low power radios, operating at 2.5V and 30 mA, with power efficiency of approximately 5% are E = 7.910-7 J/bit and  = 7.3 nJ/(bit-m4). These values are valid for 2-FSK non-coherent reception, with noise power of Np= 67dBm, at a symbol rate of 1/Tb = 10 kb/s and a bit error rate of 10-3.

You may disregard the receiver energy needed at destination because it is fixed regardless of the number of intervening relays.

Sol:

The total energy consumed for direct transmission,  Edirect = 2E +  Nxrx  (x = 2 or 4 depending on the order of power drop-off).

For the multi-hop scenario, a bit is transported to the destination via N hops. Hence, there are N transmissions and N–1 receptions, each over a distance of r. The total consumed energy is

Emultihop = NE + Nrx + (N–1)E

= (2N–1)E + Nrx.

The condition for which the direct transmission consumes less energy per bit than multi-hop transmission is,

Edirect ≤ Emultihop

2E +  Nxrx ≤ (2N–1)E + Nrx

N rx(Nx–1 –1)/(2N–3) ≤ E/

Define F(N,r) = E/ - N rx(Nx–1 –1)/(2N–3), the plots below give the region in the r - N plane where direct communication is better than multi-hop.

From the plots it can be seen that direct transmission is more favorable for second order power drop-off, as would be expected.

10.7 Dynamic modulation scaling revisited

Redo Example 10.12 taking energy consumed by the receiving radio, CR, into account. Assume CR = 12nJ.

Sol:

Ebit = [CS  f(b)+CE+CR]/b.

For CR = 12nJ:

| Range | b = 1 | b = 2 | b = 4 | b = 8 |
| --- | --- | --- | --- | --- |
| R/2 | 27.75 | 20.25 | 17.25 | 25.9 |
| # R | 39.0 | 31.5 | 39.8 |  |
| 2R | 219 | 211.5 | 401 |  |

10.8 Network lifetime

The lifetime of any network depends on the power dissipated. Using relaying rather than direct transmission extends the lifetime, as well as giving frequency re-use benefits. Let ES be the energy needed to sense a bit, ER that to receive a bit and ET that to transmit a bit over some distance d. Assume second order power loss with the distance d. The parameters associated with the node energy are as follows: 11 represents the energy consumed by transmitter electronics per bit, 2 accounts for the energy dissipated in transmit op-amp, 12 is the energy consumed by receiver electronics per bit and 3 is the energy cost of sensing a bit.

(a) Model ET, ER and ES in terms of the  parameters. (Note that the 2 depends on the propagation loss.)

(b) If the relay rate is r b/s, what is the power consumed by a relay, Prel? The function of a relay is to receive data and then to transmit them x meters onwards. Denote 11+12 by 1.  Assume the values for 1 and 2 are 180 nJ/bit and 10 pJ/bit/m2 respectively.

Figure 10.6  Communication relays

(c) If P-1 relays are introduced between S and T as illustrated in Figure 10.6, then what is the overall rate of dissipation, Plink?

(d) We are given that there are P-1 relays between S and T and the distance separating the two ends is D. Calculate the set of hop distances among the relays that will minimize Plink.

(e) Let the point source radius be dS and the distance to the fusion center from the source be dF.  Evaluate the minimum energy dissipated in the entire network. Use

Plink(D)³(2Dα1α2−α12)r.

(f) For the deployment of N sensor nodes, the active lifetime is bounded by:

L≤NE2(dF−dS)α1α2−α12+α3r.

Calculate the initial energy, E, in each node needed to support the maximum lifetime of 185 days for the deployment of 100 nodes. The fusion center is 40 m from the source and the source radius is 3 m. Assume the relay rate to be 15 b/s and that the cost of relaying data dominates.

Sol:

(a) ET = 11 + 2 d2, ER = 12, and ES = 3.

(b) Prel(d) = (1 + 2 d2)r.

(c) Plink(D) = -12 + i=1PPrel(di),where  -12 accounts for the fact that node S does not spend any energy receiving.

(d) The solution to this problem can be either done intuitively or rigorously. The rigorous proof involves the application of Jensen’s inequality to the strictly convex function Prel(di). But intuitively, one can observe that to minimize Plink(D) one has to minimize the summation of Prel(di). From geometry it can be seen that this minimum happens only when all the relays are equally spaced over the span D. Hence, the hop distance that minimizes Plink(D) is d1 = d2 = …=dp = D/P.

(e) Let dF-dS=d, Pnw  Plink(d) + PS, where PS is the power consumed while sensing. Hence the minimum energy dissipated in the entire network per second is,

Pnw=(2dα1α2−α12)r+α3r

(f) Since cost of relaying data dominates, the maximum activity lifetime is given by:

Lmax»NE2r(dF−dS)α1α2

On substituting the given data, E = 9.92 mJ/bit.

10.9 Best energy route

Figure 10.7 Minimum energy path

Find the minimum energy path in Figure 10.7 from source S to destination D. The graph shows the connectivity with associated energy cost.

Sol:

The routes SF-FD and SB-BD are the least hop distance routes from the source to the destination. But the costs of choosing these routes are 5.5 and 4 respectively. For this particular example, the optimal path is SA-AF-FD with the cost of 3.25.

10.10 Traveling salesman problem

The energy mule example is an instance of the well-known traveling salesman problem. It is not always possible to use brute force to solve the equations. Two algorithms found useful for large networks are:

(i)   Nearest neighbor algorithm: Begin at the starting node. Whenever at a node pick the next nearest unvisited node or neighbor. End at starting node.

(ii) 	Cheapest Link Algorithm:

Step 1 - Select an edge with the cheapest weight. Mark that edge.

Step 2 - Select the next cheapest unmarked edge unless:

  - The new edge completes a circuit,

  - The new edge results in three marked edges emerging from a single node.

Step 3 - Repeat Step 2 until the Hamilton circuit (closed loop through a graph that visits each node exactly once) is complete.

For Figure 10.4, find the cheapest route using (i) the nearest neighbor algorithm and (ii) the cheapest link algorithm. Comment on how well they do compared to exhaustive search. Assume the starting point to be G.

Sol:

By the nearest neighbor algorithm, the cheapest route is GB-BA-AC-CG. Hence, the weight along this path is 21.2. We know that brute force resulted in the optimal weight of 20.6. Hence, the nearest neighbor algorithm costs 2.91% extra energy. Incidentally in this particular example, the cheapest link algorithm results in the same route i.e. GB-BA-AC-CG.

Nearest neighbor algorithm:

  - Starting from G go to B, from B go to A, from A the nearest unvisited neighbor is C and from C complete the Hamilton circuit by going back to G.

Cheapest link algorithm:

- Pick the cheapest edge i.e. AB.

- Pick edge BG.

- Jump edge BC as it will result in three edges emerging out of B.

- Pick edge AC.

- Jump edge AG as it will result in three edges emerging out of A.  (It also would complete the closed loop ABG).

- Pick edge CG (Hamilton circuit is complete).

- Calculate the weight of the circuit = 21.2.

- The circuit is G-C-A-B-G.