7.1 SIR in a cellular network with reuse factor of 7.

A cellular network with a frequency reuse factor of 7 is depicted in Figure 7.12 where cells with the same letter share the same set of frequencies. In this system, mobiles communicate with base stations through two 25-kHz simplex channels, and a total of 66 MHz of bandwith is available.

- How many mobiles can each cell support in this system?

- Using the same assumptions as in example 7.1, find the worst signal to interference ratio that the base station can experience resulting from two users in different cells.

- Compute the worst signal to interference ratio that the mobile can experience resulting from two base stations.

Figure 7.12 A cellular system with reuse factor 7.

Sol:

- Bandwidth allocated to each mobile = 2.5 kHz2 = 5.0 kHz.

Total bandwidth available = 66 MHz.

Total number of mobiles that can be supported = 66,000/5.0 = 1,320

With a reuse factor of 7,

Number of mobiles each cell can support = 1,320/7  188.

- The worst case interference occurs when two mobile users’ locations are as given in the figure below. Their distances to the base station are

r1=523a,   r2=233a

The resulting signal interference ratio is

SIR=r14r24=169=22.3 dB

Note that this is not actually the worst possible SIR, since one could have potentially up to seven mobiles, all in the worst case locations in the surrounding “A” cells.  This would reduce the SIR by a factor of 7, or roughly 8 dB.

Scenarios for the worst signal to interference ratio at the base station and the mobile.

- The worst signal to interference ratio experienced by a mobile is also shown in the figure. The resulting SIR is the same as in (b).

7.2 Linear Prediction

Consider a zero mean stationary random process x(n) that has the following correlation function:

RX(n)=1n=00.8n=10.5n=20.3n=30any other n

Linear predictors with two and three delay taps are shown in Figure 7.13.

(a) Determine the coefficients w1 and w2 that minimize the variance of output y(n) in the predictor given in Figure 7.13(a).  What is the variance of output sequence?

(b) Determine the cofficients a1, a2, and a3 that minimize the variance of output y(n) in the predictor given in Figure 7.13(b). Is the variance of output sequence smaller than that in (a)?

Figure 7.13 Two linear prediction filters with two and three delay taps.

Sol:

- With two coefficients, the output y(n) can be expressed as follows

y(n)=x(n)+w1x(n−1)+w2x(n−2)

Compute its variance:

E[y2(n)]=1+w12+w22+1.6w1+w2+1.6w1w2

Next, find thew1 and w2 that minimize the variance:

∂σy2∂w1=2w1+1.6w2+1.6=0

∂σy2∂w2=2w2+1.6w1+1.0=0

Therefore,

w1=−1.111,   w2=0.389,   σy2=0.306

- With three coefficients, the output can be written as follows:

y(n)=x(n)+w1x(n−1)+w2x(n−2)+w3x(n−3).

Compute the variance of y(n):

σy2=1+w12+w22+w32+1.6w1+w2+0.6w3+1.6w1w2+w1w3+1.6w2w3.

Find w1, w2, and w3 that minimize the variance by taking partial derivatives and solving the resulting system of equations:

w1=−1.182,   w2=0.591,   w3=−0.182,   σy2=0.296.

The resulting variance is smaller than that in (a).

7.3 MIMO equalization

A discrete time two by two MIMO system is shown in Figure 7.14 where s1(n), s2(n) are source sequences, and y1 and y2 are measurement sequences. The channel transfer functions are:

h11(z)=1+az−1,  h22(z)=1+bz−1,  h12(z)=cz−1,  h21(z)=dz−1.

Besides the interference from the other source, the measurements are corrupted by the white Gaussian noise sequences v(n) with zero mean and variance σv2.

Figure 7.14 A two by two MIMO system.

- We define the vectors

y=y1(n)y2(n)y1(n−1)y2(n−1),  s=s1(n)s2(n)s1(n−1)s2(n−1)s1(n−2)s2(n−2),v=v1(n)v2(n)v1(n−1)v2(n−1).

Verify that y can be expressed in terms of s and v as

y=Cs+v

- Assume the random sources s1 and s2 are stationary and white. Their mean and covariance are given by

E[s(n)]=00,  Rs=E[s(n)sH(n)]=ss1200ss22

Find the covariance matrix of y defined as Ry=E[yyH].

- We estimate the source from measurements y based on the following scheme:

sÙ(n)=sÙ1(n)sÙ2(n)=Wy

in which Wis a 2 by 4 matrix. Determine W in terms of C, Rs, and σv2 such that the mean square error of estimation is minimized.

Sol:

- The output y(n) can be expressed as follows.

y=y1(n)y2(n)y1(n−1)y2(n−1)=s1(n)+as1(n−1)+ds2(n−1)s2(n)+bs2(n−1)+cs1(n−1)

- Compute the convariance matrix of y.

Ry=E[yyH]      =CRs000Rs000RsCH+σv2I

in which I is a 4 by 4 identity matrix.

- To determine the minimum mean square error estimator for s1(n) and s2(n), we first compute

Rsy=E[syH]=Es1(n)s2(n)y1(n)y2(n) y1(n−1)y2(n−1)      =Rs00

Then the estimator is given by

Therefore

W=Rs00CRs000Rs000RsCH+σv2I−1

7.4  Maximal ratio combining

Consider the system shown in Figure 7.15 where the signal s is coherently sensed by two receivers. The measurements y1 and y2 can be expressed as follows

y1y2=h1h2s+v1v1

where the channel gains are given by h1=aejθ1, and h2=a2ejθ2. Suppose the noise v1 and v2 are zero mean white noise with variance σv2.

Figure 7.15 MR combining

- We are to use a linear receiver y=w1y1+w2y2 to maximize the SNR at the receiver. Assume the signal s can be modeled as a white stationary process with zero mean and variance σs2. Prove that the SNR at the receiver can be expressed as

SNR=σs2||w1h1+w2h2||2σv2(w12+w22)

(b) Find the coeffecients w1 and w2 that maximize the SNR. (Hint: use  the Cauchy-Schwartz inequality, which states that the inner product of two vectors is less than  or equal to the product of two vectors’ squared norms.)

Sol:

- The output of the linear receiver can be expressed as

y=w1y1+w2y2=(w1h1+w2h2)s+(w1v1+w2v2).

Hence, the signal to noise ratio is

SNR=E(w1h1+w2h2)s2Ew1v1+w2v22=σs2w1h1+w2h22σv2||w1+w2||2.

- Using the Cauchy-Schwartz inequality,

SNR=σs2w1h1+w2h22σv2||w1+w2||2≤σs2w1+w22⋅h1+h22σv2||w1+w2||2=σs2h1+h22σv2.

Equality holds when

w1w2=ch1h2

in which h1,2 are the complex conjugates of h1,2, and c is a constant.

7.5 Array processing

Figure 7.16 Array processing

A uniform linear array consists of ten sensors in the pattern depicted in Figure 7.16. The array is steered to θ0=π3. The sensor separation is d=λ4, where  is the wavelength of interest.

- Compute the electrical angle φ for the signal coming at the designated direction of θ0.

- Suppose the following weights are used for the sensor array:

w=a(θ0)aH(θ0)a(θ0),   a(θ0)=1e−jφ⋮e−j(L−1)φ.

Plot the gain (dB) of the array (G=wHa(θ)aH(θ)w) against the incoming signal angle θ∈[−π2,π2]. (xH is the complex conjugate and transpose of  vector x.)

- Compute the signal interference ratio if another signal with the same strength as the desired signal comes from θ=π4 at half the frequency.

Sol:

- The electrical angle is

φ=2πdλsinθ0=34π.

- The gain for the array is plotted below, using the equation given in the problem.

- The electrical angle of the interfering signal is given by

φi=2πd2λsinθ=28π.

The SIR can be computed as

SIR=wHa(φ)aH(φ)wwHa(φi)aH(φi)w=1.46.

Clearly in this case filtering would be required to yield a good SIR.

Gain of a uniform linear array that is steered toward 60 degrees.

7.6 Source-sensor separation

Consider the problem of deploying sensors to monitor a certain area, in this case a square of unit area. Within the square, there is a uniformly distributed point source that we would like to measure. In other words, the source appears anywhere in the square with equal probability.  Suppose the area is divided into n2 uniform square cells, and a sensor is placed at the center of each cell. Figure 7.17 shows the setup when n2is 9.

Figure 7.17 Sensor deployment in a square.

Define d to be the distance between the point source and the closest sensor. Find the probability distribution of d in terms of n. Evaluate the average distance as a function of n.

Sol:

Because of symmetry, we only need to consider one quarter of the square cell in which the sensor is placed at the corner and the point source is uniformly distributed. This is depicted below.

One quarter of the sensing cell.

Since the source is uniformly distributed within the square area, the density function for coordinates x and y can be determined.

pX(x)=2n,     when  0≤x<1/2n0,                  otherwise

pY(y)=2n,     when  0≤y<1/2n0,                  otherwise

The distance between the sensor and the source is d=x2+y2. Its probability density function is given as follows

pD(d)=         2πnd,                               when  0≤d≤12n4ndπ2−2arccos12nd,              when  12n≤d≤12n0,                                    elsewhere

The average distance is evaluated to be

E(D)≈0.3826n

7.7 Source-sensor separation

Consider deploying n sensors in a disc of unit area. A point source appears in the disc according to a uniform distribution. Supposing the sensors are placed in the disc based on the same distribution, determine the mean distance between the point source and the closest sensor as a function of n. (Hint: use numerical evaluation.)

Sol:

Since the disc has unit area, the radius is R0=1π. Define Rs to be the distance from the uniformly distributed point source to the center of the disc. The probability density function of Rs is given by

pRs(rs)=2πrs    when   0≤rsR0

Denote by R the distance between the point source and a sensor that is randomly placed in the disc according to a uniform distribution. Given that the source is rs away from the center of the disc, the probability density function of R is as follows

pR(r|rs)=2πr,when  0≤r≤R0−rs;2θr,        when   R0−rs≤r≤R0+rs;0,elsewhere

where

θ=arccosrs2+r2−R022rsr

Given the source is rs away from the center, the probability that a sensor is less than r away from the source can be evaluated.

pR(r|rs)=πr2,when  0≤r≤R0−rs;φR02+θr2−R02sinφcosφ−r2sinθcosθ,        when   R0−rs≤r≤R0+rs;0,elsewhere

φ=arcsinrsinθR0when  R0−rs≤r≤R02+rs2;π−arcsinrsinθR0when  R02+rs2≤r≤R0+rs;

Now, suppose that n sensors are randomly placed in the disc in the same fashion. That the distance between the source and the closest sensor is r is to say that one sensor is exactly r away from the source and the other (n−1) sensors are at least r away from the source. This idea is manifested in evaluating the probability density function of Rmin, which is defined as the minimum between the source and n sensors.

pRmin(r|rs)=nfR(r|rs)[1−P(R≤r|rs)]n−1

Noticing that Rs is uniformly distributed, taking the expectation on Rs gives us the following:

pRmin(r)=ERs[pRmin(r|rs)]

This formula is numerically evaluated for different n, and the mean value of Rmin is computed.

E(Rmin)≈0.5101n

7.8 MAC schemes

Consider the following cases in conjunction with MAC schemes. Which MAC scheme appropriately describes each situation?

- Cars are waiting at the crossing for the traffic lights to turn green.

- You raise your hand before asking a question in classroom.

- Many radio stations are broadcasting at the same time.

- You are reserving a conference room for a seminar.

Sol:

- TDMA.

- Token passing.

- FDMA.

- TDMA with reservation.

7.8.9 Maximal-length sequence

Consider the feedback shift register of length 4 depicted in Figure 7.18. It consists of four flip-flops and a modulo-2 adder.

Figure 7.18 A linear feedback shift register of length 4.

- Assuming the initial state of the shift register is r1r2r3r4=1000, determine the output sequence y(n). What is the period of y(n)?

- Replace the binary symbols 0 and 1 in the sequence y(n) by levels –1 and +1. Define the autocorrelation function of y(n).

R(m)=1Nn=1Ny(n)y(n+m)

in which N is the period of the sequence.  Compute and plot the autocorrelation sequence R(m) for m=−N+1,⋯,N−1.

Sol:

Autocorrelation function of  M-sequence.

- The output sequence has period N=24−1=15.

y=000111101011001⋯

The sequence repeats itself every 15 symbols.

- The autocorrelation function is computed and plotted.

R(m)=1         when m=kN−115         otherwise

where k is any integer.

7.10 Direct sequence spread spectrum

The information bit duration of a direct-sequence spread-spectrum communication system is Tb=8.191 ms. The PN (pseudo-noise) sequence has a chip duration of  Tc=1μs. What is the processing gain of this spread spectrum system? What is the length of the shift register that is used to generate the PN sequence?

Sol:

The processing gain of the spread spectrum system is

PG=TbTc=8191

As 8191=213−1, there are 13 flip-flops in the shift register that is used to generate this PN sequence.

7.11 Ground reflection model

In a two-ray ground reflection model, the transmitter and receiver are placed ht and hr above the ground, as shown in Figure 7.19. Assume the wave obeys the free space propagation law, and is given by

E(d,t)=E0d0dcos2πfc(t−dc)

in which E0 is the field at reference distance d0. Note that the phase is a function of both time and traveling distance. The field at the receiver consists of the direct line of sight component ELOS and the ground reflection component ER.

Figure 7.19 Two-ray ground reflection model.

- Assume perfect ground reflection, in other words, at the point of reflection:

ER=−EI and α=β,

where EI is the electric field of the incident wave.  Determine the distances traveled by the line of sight ray and the reflected ray from the transmitter to the receiver. Show that the difference of the two distances can be approximated as follows, given d>>ht,hr:

Δd=dR−dLOS≈2hthrd.

- Show that the field at the receiver can be expressed as

ETOT(t)=E0d0dLOScos2πfct−dRc+Δθ−E0d0dRcos2πfct−dRc.

Find the phase shift Δθ due to the difference in distance traveled.

- Prove that the envelope of the total received field is approximately given as

|ETOT(t)|≈2E0d0dsinΔθ2.

- For small Δθ, we can use sinΔθ2≈Δθ2. Show that the power propagation in the two-ray ground reflection model obeys the following law:

Pr=|ETOT|2~1d4.

Sol:

- Simple geometry yields

dLOS=d2+(ht−hr)2,     dR=d2+(ht+hr)2

Given d>>ht,hr, we have the following

dR−dLOS=d2+(ht−hr)2−d2+(ht+hr)2                =[d2+(ht−hr)2]−[d2+(ht+hr)2]d2+(ht−hr)2+d2+(ht+hr)2                ≈4hthr2d                =2hthrd

- The field at the receiver is composed of ELOS and ER.

ETOT(t)=ELOS+ER             =E0d0dLOScos2πfct−dLOSc−E0d0dRcos2πfct−dRc             =E0d0dLOScos2πfct−dRc+Δθ−E0d0dRcos2πfct−dRc

in which

Δθ=2πfc(dR−dLOS)c≈2πfchthr(dc)

- Use the approximation dLOS≈dR≈d when d>>ht,hr.

ETOT(t)=E0d0dLOScos(θ+Δθ)−E0d0dRcosθ             ≈E0d0dcos(θ+Δθ)−cosθ             =E0d0d[(cosΔθ−1)cosθ−sinΔθsinθ]

where θ=2πfc(t−dLOSc). The envelope of the total field can be computed.

|ETOT(t)|≈E0d0d(cosΔθ−1)2+(sinΔθ)2=E0d0dsinΔθ2

- Applying sinΔθ2≈Δθ2 to (c), we have

Pr=|ETOT|2≈E0d0Δθ2d2=πfchthrE0d0d2c2~1d4

7.12 Little’s Theorem

A packet arrives at the server every K seconds. Each packet require 2K3 seconds for transmission and K2 seconds for processing. What is the average number of packets in the server?

(Hint: Little’s theorem states that the average number of packets in the system N, the average packet arrival rate λ, and the average time a packet spends in the system T are related by the formula: N=λT.)

12. Little’s Theorem

This is a straightforward application of Little’s theorem.

N=λT=1K2K3+K2=76

7.13 Aloha

In a MAC system, users transmit equal length packets through a shared channel. The transmission time for each packet is T seconds. A collision occurs when two or more packet transmissions overlap in time, and a transmission is considered successful if there is no collision during transmission. An example of the packet transmission is given in Figure 7.20, where white packets are successfully transmitted and shaded packets collide. The pure Aloha protocol is used for the media access control. The packet is transmitted immediately after its arrival. If a collision occurs, the packet is delayed by a random amount of time and transmitted again. To simplify the analysis, assume the scheduling point process, which includes the generation of new packets and the retransmission of previously collided packets, is a Poisson process with rate g.

Figure 7.20 Packet transmission in a shared channel.

- Consider that a packet is scheduled to be transmitted at time t. Find the period during which no other packet can be scheduled for transmission for this packet to be transmitted successfully. This period is called the vulnerable period.

- Determine the probability of successful transmission.

- What is the throughput of the system?

- Instead of a pure Aloha protocol, a slotted Aloha protocol is used, in which packets are sheduled for transmission only at multiples of T seconds. In other words, the slot size is T, and packets can only be transmitted at the boundaries of slots. What is the throughput of this system?

- Find the value of gT that maximizes throughput in regular and slotted Aloha and compare the two values.

Sol:

- The vulnerable period is the interval [t−T,t+T) during which no other packet transmission is allowed.

- As the duration of the vulnerable period is 2T, during which there should be no packet scheduled, the probability of successful transmission is

Psuc=e−2gT

- The throughput of the system is

S=gPsucT=gTe−2gT

- If slotted Aloha is used, the vulnerable period becomes [t−T,t). The throughput is given by

S=gPsucT=gTe−gT

- Denote u=gT. To find the u to maximize the throughput, we take the derivative and solve for u.

∂ue−2u∂u=e−2u−2ue−2u=0     ⇒     upure=12∂ue−u∂u=e−u−ue−u=0          ⇒     uslotted=1

Therefore, gslotted=1T>gpure=1(2T) (Sslotted=e−1>Spure=(12)e−1), which indicates that slotted Aloha can support a higher traffic rate than pure Aloha.